| ********** PREFLIGHT *********** # REQUIRED DOCUMENTS ARROW MAGNETOS OFF MASTER SWITCH OFF HYDRAULIC ON LANDING GEAR DOWN FLAPS UP ###### GUST LOCK REMOVED LEFT MAIN GEAR/TIRE/BRAKE CHECK # LEFT WING/TIEDOWN CHECK LEFT TIP TANK SUMP DRAIN # LEFT FLAP/AILERON CHECK # LEFT STATIC PORT CHECK FUSELAGE/ANTENNAS CHECK STABILIZER/ELEVATOR/RUDDER CHECK RIGHT STATIC PORT CHECK AUX. FUEL LEVEL & CAP CHECK RIGHT FLAP/AILERON CHECK RIGHT TIP TANK LEVEL/SUMP DRAIN PITOT TUBE CHECK STALL SWITCH CHECK RIGHT WING/TIEDOWN CHECK MAIN FUEL LEVEL & CAP CHECK MAIN FUEL SUMP DRAIN GASCOLATOR SUMP DRAIN RIGHT MAIN GEAR/TIRE/BRAKE CHECK ### RIGHT ENGINE COMPARTMENT CHECK RIGHT COWL SECURE ### PROP/SPINNER CHECK AIR FILTER CHECK HYDRAULIC RESERVOIR CHECK ENGINE OIL CHECK LEFT ENGINE COMPARTMENT CHECK LEFT COWL SECURE NOSE GEAR ASSEMBLY/CHOCK CHECK | # ********** ENGINE START********* PREFLIGHT COMPLETE SEATS/SEAT BELTS SECURE FUEL/TIP TANKS OFF FUEL MAIN BRAKES SET COWL FLAPS OPEN FUEL PUMP ON CARB. HEAT OFF MIXTURE RICH PROP FULL FWD. THROTTLE OPEN 1/8” TRIM SET FLAPS UP LANDING GEAR DOWN HYDRAULIC POWER ON MAGNETOS BOTH MASTER SWITCH ON PRIME 2 (COLD) PROPELLER CLEAR OIL COOLER OPEN STARTER ENGAGE THROTTLE 1000 RPM OIL PRESSURE CHECK AMMETER CHECK ## FUEL PUMP OFF FUEL PRESSURE CHECK ********** BEFORE TAXI ********** MIXTURE RICH AVIONICS MASTER ON RADIOS ON/SET NAVIGATION AIDS SET TRANSPONDER STANDBY ALTIMETER SET DIRECTIONAL GYRO SET MARKER RECEIVER TEST GPS/NAV CONTROL LAMPS TEST NAV LIGHTS CHECK LANDING LIGHTS AS REQ’D PARKING BRAKE RELEASE BRAKES TEST | ******** BEFORE TAKEOFF******** ### PARKING BRAKE SET FUEL SELECTOR MAIN COWL FLAPS OPEN FUEL PUMP ON MIXTURE RICH HYDRAULIC POWER ON THROTTLE 1700 RPM CARB. HEAT CHECK MAGNETOS (175/50) CHECK ENGINE INSTRUMENTS CHECK AMMETER CHECK SUCTION GAUGE CHECK THROTTLE 2000 RPM PROP CYCLE 2X THROTTLE IDLE/1000 CIRCUIT BREAKERS CHECK TRIM TAB SET DIRECTIONAL GYRO SET PITOT HEAT AS REQ’D FLAPS CYCLE/SET ENGINE PRIMER LOCKED FLIGHT CONTROLS FREE SEAT BELTS SECURE CANOPY LATCHED TRANSPONDER ALT ANTI-COLLISION LIGHTS ON LANDING LIGHTS ON AUTOPILOT OFF CROSSWIND CHECK PARKING BRAKE RELEASE **** AIRSPEEDS (MPH / KNOTS) **** BEST ANGLE (VX) 70 / 61 BEST GLIDE 80 / 70 BEST RATE (VY) 95 / 82 GEAR EXTENSION (VLO) 100 / 87 FLAP EXTENSION (VFE) 100 / 87 DESIGN MANEUVERING (VA)------------ 124 / 108 MAX. STRUCTURAL CRUISE (VNO)---- 160 / 139 NEVER EXCEED (VNE)---------------------- 190 / 165 |
| --- | --- | --- |

| # ************* CLIMB ************* ## COWL FLAPS OPEN ### LANDING GEAR UP/LOCK FLAPS UP PROP (2600 RPM / 1 MINUTE) 2300 RPM THROTTLE FULL LANDING LIGHTS OFF HYDRAULIC POWER OFF FUEL PUMP OFF ************ CRUISE ************* ## COWL FLAPS CLOSED THROTTLE SET PROP SET MIXTURE LEAN TRIM SET ENGINE INSTRUMENTS CHECK DIRECTIONAL GYRO SET ******* BEFORE LANDING ******** FUEL SELECTOR MAIN CARB. HEAT AS REQ’D MIXTURE RICH PROP FWD. HYDRAULIC POWER ON LANDING LIGHTS ON LANDING GEAR DOWN/LOCK FUEL PUMP ON AUTOPILOT OFF SEAT BELTS SECURE FLAPS SET CROSSWIND CHECK ******** AFTER LANDING ******** CARB. HEAT OFF FLAPS UP FUEL PUMP OFF MIXTURE LEAN COWL FLAPS OPEN PITOT HEAT OFF TRANSPONDER STBY ANTI-COLLISION LIGHTS AS REQ’D LANDING LIGHTS------------------------AS REQ’D | *********** TIE-DOWN ************ BRAKES SET THROTTLE 1000 RPM AVIONICS MASTER OFF ELECTRICAL ITEMS OFF PROP HIGH (OUT) MIXTURE IDLE CUT-OFF MAGNETOS OFF BATTERY / GENERATOR OFF BRAKES OFF MIXTURE NEUTRAL COWL FLAPS CLOSED VENTS CLOSED GUST LOCK INSTALLED CANOPY LOCKED KEYS IN POCKET COVER INSTALLED PITOT / STATIC / INTAKES COVERED WHEELS CHOCKED TIE DOWNS SECURE PROPELLER HORIZONTAL ********* MAINTENANCE ********* TIRE PRESSURE NOSE-----------------------------------------------------30 PSI MAINS----------------------------------------------------25 PSI STRUTS LOADED EXTENSION-------------------------1-1/2 INCH HYDRAULIC FLUID TYPE-----------------------------------------MIL-H-5606A/G ENGINE OIL QUANTITY--------------------------------------10 QUARTS MINIMUM ------ 6 QUARTS GREASE AIRFRAME--------------------------------AEROSHELL #6 HARTZELL PROP-----------------------AEROSHELL #5 PROPELLER A-38 BEARING-----RHEOLUBE 374A 2003 by Ripley Quinby III Non-commercial reproduction at will | *********** EMERGENCY ********** ******* GEAR EXTENSION ******** ## HYDRAULIC POWER OFF EMERGENCY GEAR HANDLE DOWN HYDRAULIC POWER (GEAR DOWN)---ON ****** ENGINE POWER LOSS ****** ## ESTABLISH BEST GLIDE 80 MPH FUEL SELECTOR FULLEST TANK CARB. HEAT ON MIXTURE RICH MAGNETOS BOTH FUEL PUMP ON ENGINE PRIMER LOCKED ****** POWER OFF LANDING ****** ## MIXTURE IDLE CUT-OFF MAGNETOS OFF FUEL SELECTORS OFF MASTER SWITCH OFF HYDRAULIC POWER OFF HYDRAULIC CUT-OFF ON SEAT BELTS SECURE CANOPY UNLOCK ******** ELECTRICAL FIRE ******** ## CABIN HEAT / DEFROSTER OPEN MASTER SWITCH OFF ********** ENGINE FIRE *********** ## CABIN HEAT / DEFROSTER CLOSED FUEL SELECTOR OFF MIXTURE IDLE CUT-OFF THROTTLE CLOSED FUEL PUMP OFF HYDRAULIC POWER OFF HYDRAULIC CUT-OFF ON # ****** ELECTRICAL FAILURE ****** GENERATOR SWITCH OFF LOAD SHEDDING CHECK CIRCUIT BREAKERS RESET GENERATOR SWITCH ON POWER NOT RESTORED GEN. OFF |
| --- | --- | --- |