1.0 General

This document describes the electrical operation of the General Technics Mark 1 rebreather. The Mark 1 rebreather incorporates full triple redundancy in all electrical systems. Failure of one or all components within up to two of the three oxygen sensor and delivery modules will have no effect upon the ability of the rebreather to supply full maximum U.S. Navy standard oxygen requirement at any depth for which the equipment is qualified.

1.1 User Requirements

It is assumed that the end user is fully qualified to operate and 		repair the rebreather.

1.2 Reference Documents

The following documents are hereby incorporated by reference:

Electrical Schematic

Gas Schematic

Mechanical Assembly

2.0 Overview

All critical electrical circuits within the rebreather are triply redundant, i.e.; there are three independent power, sensor, gas delivery, interlock, and system monitor circuits. Failure of any or all components within up to two of the three critical circuits will have no effect upon the remaining circuit(s).

The rebreather display circuit is not independently redundant, although failure of up to two of its three power supplies and signal inputs will not effect the operation of the rebreather or the validity of the remaining display(s).

3.0 Circuit Description

3.1 Power Supply

The rebreather is powered by three rechargeable 9 volt Lithium-ion   	batteries, designated B2 on the electrical schematic.

The batteries are wire OR’d via diode D2 at the hermetic Microdot 	charger input connection. Complete failure of any battery is isolated via 	D2 from affecting the surviving circuit(s).

Battery power is distributed via the P-channel MOSFET within U1, 	which is normally maintained in the non-conducting state via gate pull-up 	resistor R32. Closure of normally open magnetic reed switch SW1 to 	chassis ground pulls the gate of the P-channel MOSFET low, supplying 	power to the sensor circuit.

In the off condition any drain to source leakage of U1 is shunted to 	ground via R3.

In the on condition the B2 battery power is made directly available 	to the sensor circuit components and oxygen solenoid, and to the display 	module via current limiting resistor R1.

Power up of the sensor circuit applies battery voltage to the N-	channel gate of U1 via resistor R2. This allows oxygen sensor B1 to 	discharge at a low (nominal 2.4 mA) rate via R4.

3.2 Oxygen Sensor

Resistor R4 sets the response time of oxygen sensor B1, which is a commercially available type DA13 zinc-air cell.

Reduction of the ambient oxygen partial pressure available to sensor B1 will not be detected until the residual oxygen within the cell is consumed. This time constant ranges from a value of 2.0 seconds when R4 is 220 ohms to approximately 5 seconds when R4 is 470 ohms. The response time increases logarithmically to approximately 20 seconds when R4 is increased to 1K ohm. The 560 ohm value of R4 is a compromise between response time (based upon the rebreather gas volume) and sensor lifetime. The sensor has a capacity of 260 mAh, which translates to a lifetime of  0.260 Ah/(1.25 V/560 Ohms) = 116 hours maximum.

The voltage output of sensor B1, which is dependent upon the 	partial pressure of the oxygen available to it, is distributed via R5 and R11 	to the sensor comparator U2-A and sensor amplifier U2-B.

3.2.1 Sensor Comparator U2-A

Comparator U2-A is one of four voltage comparators within 			the National Semiconductor LM615. U2-A is configured as a self-			referenced non-inverting comparator, wherein the input from R5 is 			presented to the non-inverting input directly via R6, and to the 			inverting input via R8. The 	input from R8 charges C1 to the sensor 			voltage after time constant ((R1+R8)xC1), or approximately 4.75 			seconds.

Resistor R7 in conjunction with R5 sets the hysteresis of the 			comparator at 0.01% of the supply voltage (0.9 mV for a fully 			charged 9 volt supply).

Resistors R9 and R10, in parallel with the leakage of C1, act as a sensor voltage divider on the inverting input of the comparator. This ensures that the comparator output will remain stable during the slow transitions of the sensor output, and to ensure that the steady state condition of the input is below the hysteresis setting of the comparator.

The output of the comparator is open collector, and is 				normally pulled to the positive supply rail via R14. This is 				considered logic HIGH. The output is current limited via R60 and 			logically inverted at U3, a CD4069 hex inverter with active CMOS 			outputs. All inverter inputs and outputs are connected in parallel for 			redundancy.

A steady state sensor condition yields a logic HIGH from the sensor comparator and a logic LOW from the outputs of inverter U3.

As the oxygen partial pressure at sensor B1 drops, the non-			inverting input at the sensor comparator drops below the inverting 			input voltage due to the time delay of R8 and C1. This causes the 			output of comparator U2-A to go LOW, and the outputs of inverter 			U3 to go HIGH.

3.2.2  Sensor Amplifier U2-B

Sensor B1 voltage is applied to the non-inverting input of 			U2-B via current limiting resistor R11. The non-inverting direct 			current gain of this stage is (1+(R13/R12))=4.03. Capacitor C2 with 			output pull-up R15 acts as a low pass filter to set the high 				frequency corner of the amplifier at (1/(R15xC2)) = 212.8 Hertz to 			prevent oscillation.

The output of U2-B goes to the display module for oxygen 			partial pressure indication, and to a window comparator consisting 			of U2-C, U2-D, and voltage reference U2-E.

3.2.3  Window Comparator

Amplified sensor voltage is fed to the window comparator 			stages via R16 and R28. The resistors in conjunction with positive 			feedback resistors R17 and R27 provide 0.037% hysteresis (0.33 			mV at nominal 9 volt supply) to ensure that the comparators do not 			oscillate during slow transitions.

Upper comparator U2-C receives a nominal 4.055 volt 			reference at its inverting input via resistor R19, and lower 				comparator U2-D receives a nominal 5.437 volt reference via R26.

These reference voltages are generated at programmable 			reference source U2-E, which is fed through current limiting resistor 		R21.

Reference U2-E establishes a 1.244 volt difference between 			pins 8 	and 9, setting the current through R22 at (1.244/27,000) = 			46 uA. Resistor R23 sets the voltage at pin 8 to (46 uA x 91,000) = 			4.186 volts, which in addition to the 1.244 volt reference, yields a 			pin 9 output of 5.43 volts.

This voltage, designated VHI, is divided by R24 and R25 to 			yield the low reference, VLO, of 4.055 volts nominal.

The total current through the reference is set by R21 at

((9-5.43)/10,000) = 0.36 mA.

VHI and VLO are applied to the window comparator, where 			they set the upper and lower limits of acceptable levels of oxygen 			partial pressure.

The window comparator open collector outputs are wire 			OR’d together and pulled up via resistor R20. The output of the 			window comparator is logic LOW if the output of sensor B1 is less 			than (4.055/4.03) = 1.006 volt, or greater than (5.437/4.03) = 1.349 			volt. These represent conditions of sensor wear-out and oxygen 			overpressure, respectively.

For sensor B1 outputs between approximately 1 volt and 			1.35 volt (linear response region), the output of the window 				comparator is logic HIGH, and is fed to the solenoid drive logic U4.

A logic LOW condition of the window comparator is indicated 		by the illumination of the sensor failure LED on the display module.

3.2.4  Solenoid Drive Logic U4

Integrated circuit U4, a CD40107, consists of dual two input 			NAND gates driving dual inverters. The inverter outputs drive the 			gates of open drain N-channel MOSFETs. All dual functions are 			wired in parallel for redundancy, and to increase the current sink 			capability of the MOSFET output stages.

When the sensor assembly detects a decreasing oxygen 			condition the outputs of inverter U3 go HIGH. Simultaneously, the 			window comparator output is HIGH if, and only if, the sensor is 			within the linear range.

These HIGH outputs are presented to the NAND gate 				inputs, yielding a logic LOW from the NAND gate if, and only if, the 			sensor is requesting oxygen and its request is valid.

The LOW output of the NAND gate is inverted to a HIGH 			state and used to turn on the N-channel MOSFET. This presents a 			low impedance path to the oxygen valve solenoid, which is thereby 			energized.

The resulting flow of oxygen from the solenoid valve across 			sensor B1 causes the output of the sensor comparator U2-A to go 			HIGH, the outputs of inverter U3 to go LOW, and a LOW condition 			from the NAND gate of U4. This turns off the U4 MOSFETs, closes 			the solenoid valve, and stops the flow of oxygen.

Steering diodes D1 and D6 across the solenoid coil 				suppress inductive kickback during turn off of the solenoid.

Sensor B1 may be in such a condition that the output of 			sensor comparator U2-A is still valid, but the window comparator 			may disallow oxygen flow due to an out of tolerance sensor. In this 			case the NAND gate inputs will still cut off the solenoid, effectively 			removing the out of 	tolerance sensor from action. This capability is 			triply redundant and independent of the other sensor circuits, as 			follows:

Comparator U2-A Output		H	H	L	L

Inverter U3 Output			L	L	H	H

Comparator U2-C, D Output	L	H	L	H

U4 NAND gate Output		H	H	H	L

U4 inverter Output			L	L	L	H

U4 MOSFET (O2 solenoid)		OFF	OFF	OFF	ON

3.3	Display Module

The wrist mounted display module contains Light Emitting 			Diodes (LED’s) and the appropriate circuitry to condition signals 			from the sensor modules. The LED’s indicate primary power ON or 			OFF, battery condition, oxygen sensor failure, and solenoid 				actuation. Additionally, a bar graph display indicates averaged 			oxygen partial pressure from all active (valid) sensors.

Integrated circuits are a National Semiconductor LM615 			quad comparator with voltage reference, and a National 				Semiconductor LM3914 bar graph display driver.

One of the comparators is used as a unity gain buffer and 			summation amplifier for the sensor output gain stages, and the 			remaining three comparators are used in conjunction with the 			voltage reference as battery charge condition indicators.

The bar graph display gives a continuous indication of 			average oxygen partial pressure.

3.3.1  Prime Power

Power for the display module is derived from the battery 			input signals at tie points A, B, and C, through current limiting 			resistor R1, via wire OR’d diodes D3, D4, and D5. These diodes 			prevent any battery failure from effecting the remaining batteries

or circuits.

3.3.2  Power On Display

Battery power is applied directly to green LED’s 12, 14, and 			16 and current limited to approximately 10 mA via resistors R55, 			R57, and R59. Illumination of these LED’s provides indication that 			the sensor circuit and oxygen delivery system associated with that 			battery is powered up.

3.3.3  Battery Condition Indicator

Integrated circuit U5-E acts as a voltage reference for the 			battery voltage comparators.

Resistor R52 provides a total current limit for the reference 			of approximately ((9-4.147)/10,000) = 0.5 mA.

U5-E maintains a voltage of nominally 1.244 volts between 			pins 8 and 9. This voltage across R53 sets a reference current of 			(1.244/39,000) = 32 uA. The total reference voltage at pin 9 is then 			((1.244 + (R54 x 32 uA) = 4.147 volts.

This voltage is applied to the inverting inputs of comparators 			U5-B, C, and D via resistor R51.

Battery voltage is applied to the non-inverting comparator 			inputs via 1.45:1 resistive dividers consisting of R40, R41, R43, 			R44, R47, and R48. Positive feedback resistors R39, R45, and R49 		provide hysteresis of 1% of supply voltage, or a maximum of 				approximate 80 mV, for comparator stability.

Battery input voltage of greater than 1.45 times the 4.147 			volt reference will maintain the comparator outputs in a HIGH state 			via pull-up resistors R42, R46, and R50. These resistors also act to 			limit the on state current of yellow LED’s 17, 18, and 19 to 				approximately 10 mA when in the on condition.

This occurs when the battery voltage input to the appropriate 		comparator drops below the reference voltage times the divider 			ratio, or approximately 6.032 volts. The illumination of the 				associated LED indicates the approaching end of useful charge for 			that battery.

3.3.4  Sensor Condition Indicator

Input battery voltage at tie points A, B, and C is also 				available to red LED’s 11, 13, and 15 via current limit resistors R18, 		R56, and R58. These resistors limit the LED current to 				approximately 10 mA.

The LED cathodes are connected at tie points D, E, and F to 		the outputs of the sensor module window comparators. A fault 			condition detected by the window comparators (sensor B1 output 			less than 1 volt or greater than 1.35 volts) causes the open 				collector output of the comparator to go LOW, illuminating the 			associated LED.

3.3.5  Average Oxygen Partial Pressure Display

The amplified sensor voltage is summed at tie points G, H, 			and I via summing resistors R29, R30, and R31. The voltage 			summation is buffered by non-inverting amplifier U5-A, whose gain 			is set to unity with resistor R35. Resistor R33 acts as a pull up for 			the open collector output of the buffer, and the result is current 			limited via R34 to the input of U6.

Capacitor C4, in conjunction with pull-up resistor R33, acts 			as a low pass filter to set the low frequency corner of the buffer 			amplifier and prevent oscillation.

Resistor R36 sets the LED bar graph current to 				approximately 12 mA, and in conjunction with R38, sets the bar 			graph VHI and VLO references at approximately 5.43 volts and 			4.03 volts, respectively. Resistor R37 supplies the VLO reference 			to the LM3914 display driver.

Light emitting diodes LED1 through LED10 inclusive are 			multicolored. Red LED’s show the extremes of the oxygen partial 			pressure range, and green the nominal range. Yellow LED’s 				illuminate to show the transition regions. These LED’s are 				illuminated one at a time depending upon the input voltage to U6 			within the range of VHI to VLO. Each LED represents a sensor 			output voltage range of (((5.437-4.03)/4.03)/10) = 0.0349 volts.

LED current is set to approximately 12 mA via R36 				according to the formula I = (12.5/R36).

Decoupling capacitor C3 prevents oscillation of U6 during 			LED turn on and transition, while R64 acts as an overall current 			limit.

3.3.6  Solenoid Actuation Indicators

Light emitting diodes LED20, 21, and 22 are forward biased 			by the battery supplies at tie points A, B, and C. Current limiting is 			provided by resistors R61, R62, and R63 respectively, with the 			cathodes of the LED’s connected to tie points J, K, and L.

The latter tie points are paralleled with their respective  			oxygen control solenoids at U4. Actuation of the solenoid, with the 			associated delivery of oxygen, is displayed by the simultaneous 			illumination of the appropriate green LED.

3.3.7  Tie Point Color Coding

Display module tie points are wire color coded as follows:

TIE POINT	FUNCTION	COLOR	LED COLOR

A		SUPPLY	BLUE		GREEN

B		SUPPLY	BLUE		GREEN

C		SUPPLY	BLUE		GREEN

D		SENSOR	GREEN	RED

E		SENSOR	GREEN	RED

F		SENSOR	GREEN	RED

G		PO2		GRAY		BAR GRAPH

H		PO2		GRAY		BAR GRAPH					I		PO2		GRAY		BAR GRAPH

J		SOLENOID	WHITE	GREEN

K		SOLENOID	WHITE	GREEN

L		SOLENOID	WHITE	GREEN

GROUND	SUPPLY	GREEN / YELLOW